| GENERAL RISKS & CONTROLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Classification (state the category of danger) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ☐ |  |  | Serious Health Hazard/Toxic |  |  |  |  |  | ☐ |  | Oxidising |  |  |  |  |  | ☒ |  |  | Gas Under Pressure |  |  |  |
|  | ☒ |  |  | Health Hazard/ Harmful/Irritant |  |  |  |  |  | ☒ |  | Flammable |  |  |  |  |  | ☐ |  |  | Acute Toxicity/ Carcinogen |  |  |  |
|  | ☐ |  |  | Corrosive |  |  |  |  |  | ☒ |  | Explosive |  |  |  |  |  | ☐ |  |  | Dangerous for the Environment |  |  |  |
| Hazard Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ |  |  | ☐ |  |  | ☐ |  |  | ☐ |  |  |  | ☐ |  |  | ☒ |  |  |  | ☐ |  |  | ☐ |  |
| Gas |  |  | Water |  |  | Mist |  |  | Fume |  |  |  | Dust |  |  | Liquid |  |  |  | Solid |  |  | Other |  |
| Route of Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ |  | ☒ |  |  |  |  | ☒ |  |  |  | ☐ |  |  | ☐ |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  | Skin |  |  |  |  | Eyes |  |  |  | Ingestion |  |  | Other (state) |  |  |  |  |  |  |  |  |  |  |
| Workplace Exposure Limits (WELs) please indicate n/a where not applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 150ppm/474mg3 8 hours |  |  |  |  |  |  |  |
| Storage & Handling |  |  |  |  |  |  |  |  |  |  |  |  | Hazard Symptoms |  |  |  |  |  |  |  |  |  |  |  |
| - Secure cylinders upright to prevent them from falling. - Store separate from oxidant cylinders stored 10meters away, and away from ignition sources. No smoking. - Keep in a cool well ventilated area, in a locked cage. - Store at least 1meter away from buildings. Ensure adequate signage is in place with firefighting equipment. - Not compatible with Oxidising agents. Protect against potential impact and puncture. - Do NOT drag cylinders when handling, always keep in upright position. |  |  |  |  |  |  |  |  |  |  |  |  | - Inhalation of vapour can cause irritation to nose and throat, headaches, nausea and vomiting. - May cause frostbite/cold burns when in contact with skin. - Cylinders are heavy, especially when full. - Substance is heavier then air and will collect at low levels. Substance may travel on declining surfaces and contaminate remote areas. - Extremely flammable liquefied gas. - Risk of explosion, accidental release through damaged cylinders or valve. |  |  |  |  |  |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  |  |  |  |  |  |  | Remedial Actions |  |  |  |  |  |  |  |  |  |  |  |
| - Store cylinders outside when not in use to ensure good natural ventilation. Wear appropriate RPE. - Use flame retardant overalls and suitable gloves to prevent frostbite and burns from contact with liquid. - Wear safety footwear and gloves when handling cylinders. Transport on site using trolleys, reduce unnecessary handling were possible. - Avoid storage and use close to excavation, drains, basements, and confined spaces where substance may accumulate. - Ensure hot works permit to work system is in place. - Keep cylinders away from sources of ignition - No smoking when in use or near storage areas. Ensure flashback arrestors are fitted. - Suitable fire extinguishers to be available i.e water, powder. - General training in the safe use, handling, to be able to identify damage and faults. Do NOT drag roll cylinder when handling. |  |  |  |  |  |  |  |  |  |  |  |  | - Evacuate the area including drains, basements, and excavations. - Eliminate all sources of ignition if safe to do so. - Stop the leak if safe to do so. - Allow for good ventilation. Ensure RPE is worn. - Absorb spill using absorbent material. And store in sealed container - - If leaking gas flame cannot be extinguished, allow to burn. |  |  |  |  |  |  |  |  |  |  |  |
| Disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste? ☐ |  |  |  |  | Skip ☐ |  |  |  |  |  |  |  | - Do not release when risk of forming an explosive atmosphere. - Waste gas should be flared in a safe place through a suitable burner with flash back arrestors. - Return back to supplier otherwise treat as hazardous waste. - Do NOT burn, incinerate, or crush cylinders. |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot ☐ |  |  |  |  | Return to Supplier ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FIRST AID |  |  |  |  |  |  |  |  |  |  |  |  | FIRE FIGHTING |  |  |  |  |  |  |  |  |  |  |  |
| - Remove victim from to uncontaminated area. Keep warm and rested. - If breathing is stopped, trained person to apply artificial respiration and call doctor. - Remove contaminated clothes. Rinse skin with warm soap water. Apply moisturising dressing. - If irritation continues seek medical advice. |  |  |  |  | - Remove contact lenses, and flush eyes with water. Repeat for 15 minutes. - If discomfort continues seek medical attention. - Ingestion is not considered a potential route of exposure. But if occurs, seek medical attention. |  |  |  |  |  |  |  | WATER ☒ |  | FOAM ☐ |  | POWDER ☒ |  |  |  |  | CO2 ☐ |  | CHEMICAL ☐ |
|  |  |  |  |  |  |  |  |  |  |  |  |  | PRECAUTIONS: Exposure to fire may cause cylinder to erupt or explode. Do NOT use water jet, use spray/mist. Do NOT extinguish a leaking gas flame unless necessary. |  |  |  |  |  |  |  |  |  |  |  |

| SITE SPECIFIC CONTROLS (TO BE COMPLETED ON SITE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activities Being Undertaken |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Persons |  |  |  | Frequency |  |  |  |  | Duration |  |  |  |  | Any specific training requirements? |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  |  |  |
| Personal Protective Equipment (PPE) Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☒ |  | ☒ |  |  | ☐ |  | ☐ |  | ☒ |  |  | ☐ |  | ☐ |
| Eye Protection | Ear Protection |  | Hard Hat |  | Footwear |  |  | Overalls |  | Visor |  | Gloves |  |  | Face Mask |  | Respirator |
| Storage Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCKED CAGE |  | BUNDED CONTAINER |  |  |  |  | PROTECTION FROM WEATHER |  |  |  | COSHH STORAGE ROOM |  |  |  |  | SEALED CONTAINER |  |
| ☐ |  | ☐ |  |  |  |  | ☐ |  |  |  | ☐ |  |  |  |  | ☐ |  |
| Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEGREGATED STORAGE |  |  |  |  |  | SEALED CONTAINERS |  |  |  |  |  |  | WASTE CARRIER DISPOSAL / RECYCLING LICENCE |  |  |  |  |
| ☐ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☐ |  |  |  |  |
| Risk Rating Following Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOW |  |  |  |  |  | MEDIUM |  |  |  |  |  |  | HIGH |  |  |  |  |
| ☐ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☒ |  |  |  |  |

| Site Specific RA Prepared by: | Si King | Position: | Contracts Director |
| --- | --- | --- | --- |
|  |  | Review Date: | 28/09/2023 |