Heritage Team Response

Date – 2/10/2024

APP/24/00199

Application site – Church House, The Pallant, Havant

Site

The site is located on the corner of The Pallant and Prince George Street within Havant Town Centre. It comprises a two-storey building, comprising several different sections. It is also within the St Faiths Conservation Area.

The property has been Grade II listed by Historic England with the following listing description:

‘House (church office). C18, with early C19 alterations. Brick, with a tile roof. Symmetrical north side of 2 storeys and attic, 3 windows. Tile roof with coved plastered eaves, 3 hipped dormers with sashes. Painted brick walling in Flemish bond, rubbed flat arches, plastered 1st floor band and plinth. Victorian sashes. The west gable, with southward extensions, includes decorated cusped bargeboards and a porch of 4 columns, with cornice, panelled frieze, 6-flush-panelled door between side windows. The long east wing has a tile roof, brick dentil eaves, red brick walls in Flemish bond, rubbed flat arches, and Victorian sashes. The plain, brick and tile, stable block at the south-west corner, is used as a shop’.

The frontage to the building is on The Pallant, whilst the rear of the site is enclosed by a historic brick boundary wall and which also includes a car park. The site complex also includes Pallant Hall which is used for community use.

Proposal

Listed building consent for replacement of 5no. ground floor windows.

The proposal is to replace these with like for like, wooden frame, sash windows. It is proposed to replace these with slim line style double glazing (6-4-6mm). The finish will be white painted windows with black architraves as existing.

Legislative and Policy Context:

When assessing the proposal, the following legislation and policy context is taken in consideration.

Section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990 ("1990 Act") requires local planning authorities to have special regard to the desirability of preserving a listed building, its setting or any features of special architectural or historic interest when considering whether to grant listed building consent.

Section 66 of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires planning authorities, when considering whether to grant planning permission for development which affects a listed building or its setting, to have special regard to the desirability of preserving the building or its setting or any features of special architectural or historic interest which it possesses.

Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires planning authorities, when considering whether to grant planning permission for development which affects a Conservation Area, to have special regard to the desirability of preserving or enhancing the character and appearance of that area.

NPPF

The revised NPPF (December 2023) sets out in Chapter 16, the core principles relating to development affecting Heritage Assets that local planning authorities should consider in making planning decisions in the following paragraphs: 195 – 214.

Local Plan

Local planning policy comprises the adopted Havant Borough Local Plan Core Strategy 2011, along with the Allocations Plan 2014. With regards to local planning policy, the following policies are relevant:

Policy CS11 of the Core Strategy states that planning permission will be granted for development that:

‘Protects and where appropriate enhances the borough’s statutory and non-statutory heritage designations by appropriately managing development in or adjacent to conservation areas, listed buildings, scheduled ancient monuments, historic parks and gardens, archaeological sites, buildings of local historic or architectural interest’.

Policy CP16 (1a) of the Core Strategy states ‘Planning permission will be granted for development that is designed to a high standard, which helps to create places where people want to live, work and relax. All development should demonstrate that its design:

‘Identifies and responds positively to existing features of natural, historic or local character within or close to the proposed development site’.

Policy DM20 of the Allocations Plan states that:

‘Planning permission will be granted for development proposals that conserves and enhances the historic assets of Havant. It also states that application which affect, or have the potential to affect, heritage assets are expected to provide a Heritage Statement that:

1. Describe the significance of the asset and its setting, using appropriate expertise

and where necessary original survey, at a level of detail proportionate to its

significance and sufficient to understand the potential impact of the proposal.

2. Sets out the impact of the development on the heritage assets and mitigation that is

proportionate to the impact and the significance of the heritage asset, including

where possible positive opportunities to conserve and enjoy heritage assets’.

Assessment

The windows proposed for replacement are located within the historic core of the building at ground floor level. An internal site visit caried out in December 2023 revealed that the windows are beyond reasonable economic repair and currently in very poor condition. As a result, the rooms in which these windows are located, are not usable as the building is used for community use.

The existing windows are timber framed with single glazing. However, whilst they are historic in character, they appear to have been altered in the past including modern frames (very simple with no internal architraves) and some glazing has been replaced with single panes of Perspex, rather than glass.

The proposal is to replace like for like with timber frames in the same sash style as the existing., finished with a white gloss paint. The only alteration would be the use of double glazing instead of single glazing. However, a slimline style of double glazing is being proposed, with a profile of 4-6-4mm, which almost replicates the appearance of single glazing. Furthermore, these windows would not feature any glazing bars, which would make the slimline double glazing even less apparent.  Further plans were submitted during the course of the application so that all the relevant details for the replacement windows are now included.

Conclusion

The use of slimline glazing within the replacement windows, would result in small degree of harm, that being less than substantial harm, at the lower end of the scale. However, the benefits of this proposal would be the ability to re-use two of the ground floor rooms for the community use building. It is considered that this benefit outweighs the small degree of harm, for which there is clear and convincing evidence in the form of the need to repair the existing windows.

With regards to the impact of the proposal on the St Faiths Conservation Area, it is considered that the proposal would not result in harm to the significance of the conservation area, as the overall visual contribution made by the building would be improved. The proposal would therefore meet the test set out in Section 72 of the Planning (Listed Buildings and Conservation Areas) 1990 and the relevant national and local planning policies.

NO OBJECTION