| GENERAL RISKS & CONTROLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Classification (state the category of danger) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ☐ |  |  | Serious Health Hazard/Toxic |  |  |  |  |  | ☐ |  | Oxidising |  |  |  |  |  | ☐ |  |  | Gas Under Pressure |  |  |  |
|  | ☒ |  |  | Health Hazard/ Harmful/Irritant |  |  |  |  |  | ☐ |  | Flammable |  |  |  |  |  | ☐ |  |  | Acute Toxicity/ Carcinogen |  |  |  |
|  | ☐ |  |  | Corrosive |  |  |  |  |  | ☐ |  | Explosive |  |  |  |  |  | ☒ |  |  | Dangerous for the Environment |  |  |  |
| Hazard Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ |  |  | ☐ |  |  | ☐ |  |  | ☐ |  |  |  | ☐ |  |  | ☒ |  |  |  | ☐ |  |  | ☐ |  |
| Gas |  |  | Water |  |  | Mist |  |  | Fume |  |  |  | Dust |  |  | Liquid |  |  |  | Solid |  |  | Other |  |
| Route of Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ |  | ☒ |  |  |  |  | ☒ |  |  |  | ☐ |  |  | ☐ |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  | Skin |  |  |  |  | Eyes |  |  |  | Ingestion |  |  | Other (state) |  |  |  |  |  |  |  |  |  |  |
| Workplace Exposure Limits (WELs) please indicate n/a where not applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5mg/m3 TWA |  |  |  |  |  |  |  |
| Storage & Handling |  |  |  |  |  |  |  |  |  |  |  |  | Hazard Symptoms |  |  |  |  |  |  |  |  |  |  |  |
| - Store in cool, well ventilated area, in original container on an impermeable floor and away from acids. - Avoid sources of ignition and heat sources, and ingress of water. - Do NOT remove labels. - Ensure container is protected from impact. - Avoid strong oxidising agents, and strong acids. - Ensure container does not have residual grease on outside of container when lifting or handling. |  |  |  |  |  |  |  |  |  |  |  |  | - In combustion emits toxic fumes. - Release into the environment causes damage to aquatic organisms. - Container may be slippery due to residual grease left on the container. - Creates slippery surface. - Ingesting substance can cause diarrhoea and stomach pains. - Extended exposure to skin can cause irritation and dermatitis. - Contact with the eyes will cause irritation. |  |  |  |  |  |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  |  |  |  |  |  |  | Remedial Actions |  |  |  |  |  |  |  |  |  |  |  |
| - Avoid leaving the substance close to sources of extreme temperature, oxidising agents, and strong acids. - Store on impermeable ground. Use bund to ensure leaks to not contaminate the ground. - Ensure container is adequately free from grease before lifting/handling. Use gloves with suitable grip, and mechanical aids were necessary. - Clean up procedure if spilt: Absorb into dry earth or sand and dispose of waste accordingly. - Do NOT eat, drink, smoke, when using. Maintain high level of hygiene. - Wear impermeable protective gloves when handling. - Wear protective eye wear. |  |  |  |  |  |  |  |  |  |  |  |  | - Stop the spill if safe to do so. - Prevent entry into drains, sewers, and watercourses. - Soak up with absorbent material. - If spilt into water, contain by a boom, then collect by skimming or absorption. - Clean the contaminated area, surface will be slippery. |  |  |  |  |  |  |  |  |  |  |  |
| Disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste? ☐ |  |  |  |  | Skip ☒ |  |  |  |  |  |  |  | Other ☒ - Place used/contaminated materials in suitable containers. - Do NOT burn or incinerate empty or full containers. - Dispose of controlled waste using licenced waste disposal company. |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot ☐ |  |  |  |  | Return to Supplier ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - FIRST AID |  |  |  |  |  |  |  |  |  |  |  |  | FIRE FIGHTING |  |  |  |  |  |  |  |  |  |  |  |
| - No applicable. May exert toxic fumes when exposed to acids and combustion. In the event seek medical attention - Remove contaminated cloths and wash with warm soap water immediately. - Seek medical attention if irritation continues. |  |  |  |  | - Remove contact lenses and flush eye immediately with water. Seek medical attention if discomfort continues. - Wash mouth out with water. Do not induce vomiting. Seek medical attention if discomfort continues. |  |  |  |  |  |  |  | WATER ☒ |  | FOAM ☒ |  | POWDER ☒ |  |  |  |  | CO2 ☒ |  | CHEMICAL ☒ |
|  |  |  |  |  |  |  |  |  |  |  |  |  | PRECAUTIONS: Combustion emits toxic fumes. Use media best suited for surroundings. |  |  |  |  |  |  |  |  |  |  |  |

| SITE SPECIFIC CONTROLS (TO BE COMPLETED ON SITE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activities Being Undertaken |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuelling of Plant & Equipment on site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Persons |  |  |  | Frequency |  |  |  |  | Duration |  |  |  |  | Any specific training requirements? |  |  |  |
| 5-10 |  |  |  | Daily |  |  |  |  | 30 mins |  |  |  |  | ☐ |  |  |  |
| Personal Protective Equipment (PPE) Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | ☐ |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Eye Protection | Ear Protection |  | Hard Hat |  | Footwear |  |  | Overalls |  | Visor |  | Gloves |  |  | Face Mask |  | Respirator |
| Storage Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCKED CAGE |  | BUNDED CONTAINER |  |  |  |  | PROTECTION FROM WEATHER |  |  |  | COSHH STORAGE ROOM |  |  |  |  | SEALED CONTAINER |  |
| ☐ |  | ☐ |  |  |  |  | ☐ |  |  |  | ☐ |  |  |  |  | ☐ |  |
| Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEGREGATED STORAGE |  |  |  |  |  | SEALED CONTAINERS |  |  |  |  |  |  | WASTE CARRIER DISPOSAL / RECYCLING LICENCE |  |  |  |  |
| ☐ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☐ |  |  |  |  |
| Risk Rating Following Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOW |  |  |  |  |  | MEDIUM |  |  |  |  |  |  | HIGH |  |  |  |  |
| ☐ |  |  |  |  |  | ☐ |  |  |  |  |  |  | ☐ |  |  |  |  |

| Site Specific RA Prepared by: | Si King | Position: | Contracts Director |
| --- | --- | --- | --- |
|  |  | Review Date: | 27/09/2023 |