| GENERAL RISKS & CONTROLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Classification (state the category of danger) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ☐ |  |  | Serious Health Hazard/Toxic |  |  |  |  |  | ☐ |  | Oxidising |  |  |  |  |  | ☐ |  |  | Gas Under Pressure |  |  |  |
|  | ☒ |  |  | Health Hazard/ Harmful/Irritant |  |  |  |  |  | ☐ |  | Flammable |  |  |  |  |  | ☐ |  |  | Acute Toxicity/ Carcinogen |  |  |  |
|  | ☐ |  |  | Corrosive |  |  |  |  |  | ☐ |  | Explosive |  |  |  |  |  | ☒ |  |  | Dangerous for the Environment |  |  |  |
| Hazard Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ |  |  | ☐ |  |  | ☐ |  |  | ☐ |  |  |  | ☐ |  |  | ☒ |  |  |  | ☐ |  |  | ☐ |  |
| Gas |  |  | Water |  |  | Mist |  |  | Fume |  |  |  | Dust |  |  | Liquid |  |  |  | Solid |  |  | Other |  |
| Route of Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ |  | ☒ |  |  |  |  | ☒ |  |  |  | ☒ |  |  | ☐ |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  | Skin |  |  |  |  | Eyes |  |  |  | Ingestion |  |  | Other (state) |  |  |  |  |  |  |  |  |  |  |
| Workplace Exposure Limits (WELs) please indicate n/a where not applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10mg/15mins |  |  |  |  |  |  |  |
| Storage & Handling |  |  |  |  |  |  |  |  |  |  |  |  | Hazard Symptoms |  |  |  |  |  |  |  |  |  |  |  |
| - Keep substance in original container tightly closed. - Avoid sources of heat and ignition, and ingress of water. - A bund area is required (110% percent capacity of container.) - Keep on impermeable ground. - Do NOT remove labels. - Protect against impact and puncture. |  |  |  |  |  |  |  |  |  |  |  |  | - Substance may cause slight skin irritation. - May cause slight eye irritation. - Ingestion may cause gastrointestinal irritation, nausea, vomiting and diarrhoea. - Harmful to aquatic organisms. - Barrels are heavy even when empty. - Inheritably combustible. |  |  |  |  |  |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  |  |  |  |  |  |  | Remedial Actions |  |  |  |  |  |  |  |  |  |  |  |
| - Wear impermeable gloves, and overalls where necessary. - Avoid contact with the eyes. Use eye protection. - Do NOT siphon by mouth. Do NOT eat, drink smoke when using, and wash hands after use. - Store on bund, on impermeable grounds. Keep spill kit close by. - Use mechanical aids, and manual handling techniques when handling. Do NOT drag, roll, or push drums. - Keep away from open flames, heat, hot surfaces, and other sources of ignition. |  |  |  |  |  |  |  |  |  |  |  |  | - Stop the spill if safe to do so. - Prevent entry into drains, sewers, and watercourses. - Soak up with absorbent material. - If spilt into water, contain by a boom, then collect by skimming or - absorption. - Clean the contaminated area, surface will be slippery. - Notify Environmental agency if spill contaminates water. |  |  |  |  |  |  |  |  |  |  |  |
| Disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste? ☐ |  |  |  |  | Skip ☒ |  |  |  |  |  |  |  | - Place used/contaminated materials in suitable containers. - Dispose of controlled waste in accordance with Duty of Care Regs (Environmental Protection Act 1990-Section34) Using a licenced waste disposal company. - Do NOT burn or incinerate drums. Other ☐ |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot ☐ |  |  |  |  | Return to Supplier ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FIRST AID |  |  |  |  |  |  |  |  |  |  |  |  | FIRE FIGHTING |  |  |  |  |  |  |  |  |  |  |  |
| Remove to fresh air. If effects persist, seek medical attention. Wash thoroughly with warm soap water. Seek medical attention if irritation occurs. |  |  |  |  | Remove contact lenses, and flush with water. If irritation continues get medical attention. Rinse mouth with water. Do NOT induce vomiting, unless instructed by a medical professional. |  |  |  |  |  |  |  | WATER ☒ |  | FOAM ☒ |  | POWDER ☒ |  |  |  |  | CO2 ☐ |  | CHEMICAL ☐ |
|  |  |  |  |  |  |  |  |  |  |  |  |  | PRECAUTIONS: |  |  |  |  |  |  |  |  |  |  |  |

| SITE SPECIFIC CONTROLS (TO BE COMPLETED ON SITE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activities Being Undertaken |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fuelling of Plant & Equipment on site |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Persons |  |  |  | Frequency |  |  |  |  | Duration |  |  |  |  | Any specific training requirements? |  |  |  |
| 5-10 |  |  |  | Daily |  |  |  |  | 30 mins |  |  |  |  | ☐ |  |  |  |
| Personal Protective Equipment (PPE) Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | ☐ |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |  | ☐ |  |  | ☐ |  | ☐ |
| Eye Protection | Ear Protection |  | Hard Hat |  | Footwear |  |  | Overalls |  | Visor |  | Gloves |  |  | Face Mask |  | Respirator |
| Storage Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCKED CAGE |  | BUNDED CONTAINER |  |  |  |  | PROTECTION FROM WEATHER |  |  |  | COSHH STORAGE ROOM |  |  |  |  | SEALED CONTAINER |  |
| ☐ |  | ☐ |  |  |  |  | ☐ |  |  |  | ☐ |  |  |  |  | ☐ |  |
| Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEGREGATED STORAGE |  |  |  |  |  | SEALED CONTAINERS |  |  |  |  |  |  | WASTE CARRIER DISPOSAL / RECYCLING LICENCE |  |  |  |  |
| ☐ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☐ |  |  |  |  |
| Risk Rating Following Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOW |  |  |  |  |  | MEDIUM |  |  |  |  |  |  | HIGH |  |  |  |  |
| ☐ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☐ |  |  |  |  |

| Site Specific RA Prepared by: | Si King | Position: | Contracts Director |
| --- | --- | --- | --- |
|  |  | Review Date: | 28/09/2023 |