| GENERAL RISKS & CONTROLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Classification (state the category of danger) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ☐ |  |  | Serious Health Hazard/Toxic |  |  |  |  |  | ☐ |  | Oxidising |  |  |  |  |  | ☐ |  |  | Gas Under Pressure |  |  |  |
|  | ☒ |  |  | Health Hazard/ Harmful/Irritant |  |  |  |  |  | ☒ |  | Flammable |  |  |  |  |  | ☐ |  |  | Acute Toxicity/ Carcinogen |  |  |  |
|  | ☐ |  |  | Corrosive |  |  |  |  |  | ☐ |  | Explosive |  |  |  |  |  | ☐ |  |  | Dangerous for the Environment |  |  |  |
| Hazard Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ |  |  | ☐ |  |  | ☐ |  |  | ☐ |  |  |  | ☐ |  |  | ☒ |  |  |  | ☐ |  |  | ☐ |  |
| Gas |  |  | Water |  |  | Mist |  |  | Fume |  |  |  | Dust |  |  | Liquid |  |  |  | Solid |  |  | Other |  |
| Route of Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ |  | ☒ |  |  |  |  | ☒ |  |  |  | ☒ |  |  | ☐ |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  | Skin |  |  |  |  | Eyes |  |  |  | Ingestion |  |  | Other (state) |  |  |  |  |  |  |  |  |  |  |
| Workplace Exposure Limits (WELs) please indicate n/a where not applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 750ppm/1060mg 15mins |  |  |  |  |  |  |  |
| Storage & Handling |  |  |  |  |  |  |  |  |  |  |  |  | Hazard Symptoms |  |  |  |  |  |  |  |  |  |  |  |
| - Store in original packaging. - Keep away from heat and sources of ignition. - General training in the safe use, handling, disposal and storage. - Store in locked COSHH store out of direct sunlight. - Avoid potential impact and puncture. |  |  |  |  |  |  |  |  |  |  |  |  | - Extremely flammable. - Container is under pressure. - Harmful by inhalation and if swallowed. - Will defat skin and prolonged contact may produce dermatitis. - Irritant to eyes. - Limited evidence of a carcinogenic (causing cancer) effect. |  |  |  |  |  |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  |  |  |  |  |  |  | Remedial Actions |  |  |  |  |  |  |  |  |  |  |  |
| - Use away from heat and ignition sources. - Do not pierce/puncture or incinerate even when container is empty. - Ensure good ventilation and avoid use in confined spaces. Do NOT eat, drink, smoke while using. Wash hands after use. - Ensure impermeable gloves are worn, wear overalls were necessary. - Wear appropriate eye protection. - Avoid prolonged contact with the substance. |  |  |  |  |  |  |  |  |  |  |  |  | - Stop the spill if safe to do so. - Clear any spillages using any available spill kit with absorbent - material. - Eliminate all sources of ignition. - Ensure adequate ventilation Do not allow entry into environment. |  |  |  |  |  |  |  |  |  |  |  |
| Disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste? ☒ |  |  |  |  | Skip ☐ |  |  |  |  |  |  |  | Other ☒ Ensure that container is empty and dispose of according local authority regulations. Do not pierce or incinerate even when container is empty. Separate waste container for aerosol cans. |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot ☐ |  |  |  |  | Return to Supplier ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FIRST AID |  |  |  |  |  |  |  |  |  |  |  |  | FIRE FIGHTING |  |  |  |  |  |  |  |  |  |  |  |
| Move exposed person to fresh air. Seek medical attention is discomfort continues. Promptly wash contaminated skin with soap or mild detergent and water. - Remove clothing is soaked through ad wash as above. |  |  |  |  | Remove clothing is soaked through ad wash as above. - Immediately flush eyes with waste while lifting the eye lid. Continue procedure for 15 minutes before seeking medical attention. - Rinse mouth thoroughly. Do not induce vomiting. Seek medical attention if irritation persist. |  |  |  |  |  |  |  | WATER ☐ |  | FOAM ☒ |  | POWDER ☒ |  |  |  |  | CO2 ☒ |  | CHEMICAL ☐ |
|  |  |  |  |  |  |  |  |  |  |  |  |  | PRECAUTIONS: May explode in fire, May flash back. Forms explosive atmosphere. Do NOT use water jet, only spray/mist. |  |  |  |  |  |  |  |  |  |  |  |

| SITE SPECIFIC CONTROLS (TO BE COMPLETED ON SITE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activities Being Undertaken |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Persons |  |  |  | Frequency |  |  |  |  | Duration |  |  |  |  | Any specific training requirements? |  |  |  |
| 5 -10 |  |  |  | Daily |  |  |  |  | 8 Hiuyrs |  |  |  |  | ☐ |  |  |  |
| Personal Protective Equipment (PPE) Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☒ |  | ☒ |  |  | ☐ |  | ☐ |  | ☒ |  |  | ☐ |  | ☐ |
| Eye Protection | Ear Protection |  | Hard Hat |  | Footwear |  |  | Overalls |  | Visor |  | Gloves |  |  | Face Mask |  | Respirator |
| Storage Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCKED CAGE |  | BUNDED CONTAINER |  |  |  |  | PROTECTION FROM WEATHER |  |  |  | COSHH STORAGE ROOM |  |  |  |  | SEALED CONTAINER |  |
| ☐ |  | ☐ |  |  |  |  | ☐ |  |  |  | ☒ |  |  |  |  | ☐ |  |
| Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEGREGATED STORAGE |  |  |  |  |  | SEALED CONTAINERS |  |  |  |  |  |  | WASTE CARRIER DISPOSAL / RECYCLING LICENCE |  |  |  |  |
| ☐ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☐ |  |  |  |  |
| Risk Rating Following Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOW |  |  |  |  |  | MEDIUM |  |  |  |  |  |  | HIGH |  |  |  |  |
| ☒ |  |  |  |  |  | ☐ |  |  |  |  |  |  | ☐ |  |  |  |  |

| Site Specific RA Prepared by: | Si King | Position: | Contracts Director |
| --- | --- | --- | --- |
|  |  | Review Date: | 28/09/2023 |