| GENERAL RISKS & CONTROLS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Classification (state the category of danger) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ☐ |  |  | Serious Health Hazard/Toxic |  |  |  |  |  | ☐ |  | Oxidising |  |  |  |  |  | ☐ |  |  | Gas Under Pressure |  |  |  |
|  | ☒ |  |  | Health Hazard/ Harmful/Irritant |  |  |  |  |  | ☐ |  | Flammable |  |  |  |  |  | ☐ |  |  | Acute Toxicity/ Carcinogen |  |  |  |
|  | ☒ |  |  | Corrosive |  |  |  |  |  | ☐ |  | Explosive |  |  |  |  |  | ☒ |  |  | Dangerous for the Environment |  |  |  |
| Hazard Type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ |  |  | ☐ |  |  | ☐ |  |  | ☐ |  |  |  | ☒ |  |  | ☐ |  |  |  | ☐ |  |  | ☐ |  |
| Gas |  |  | Water |  |  | Mist |  |  | Fume |  |  |  | Dust |  |  | Liquid |  |  |  | Solid |  |  | Other |  |
| Route of Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ |  | ☒ |  |  |  |  | ☒ |  |  |  | ☒ |  |  | ☐ |  |  |  |  |  |  |  |  |  |  |
| Inhalation |  | Skin |  |  |  |  | Eyes |  |  |  | Ingestion |  |  | Other (state) |  |  |  |  |  |  |  |  |  |  |
| Workplace Exposure Limits (WELs) please indicate n/a where not applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10mg/ 8 hours |  |  |  |  |  |  |  |
| Storage & Handling |  |  |  |  |  |  |  |  |  |  |  |  | Hazard Symptoms |  |  |  |  |  |  |  |  |  |  |  |
| - Keep shielded from weather, and humid environments. - Store away from objects that may puncture the packaging. - Cautions should be taken when manual handling due to weight of packaged substance. Use of mechanical aids is advised when dealing with large quantities. - Store away from aluminium powder as when mixed with wet cement hydrogen is produced. - Keep bulk supplies in shrink wrapped packaging were possible to prevent residual dust being released. |  |  |  |  |  |  |  |  |  |  |  |  | - Cement may have an irritating effect on moist skin due to transpiration or humidity. - Prolonged contact with wet cement may cause serious chemical burns, and can develop without pain being felt. - Eye contact with cement, wet or dry, may cause serious and potentially irreversible injuries. - Frequent inhalation of large quantities of cement dust over a longer period increases the risk of developing lung disease. - Nerve endings may be damaged prior to skin burning, hence burning may occur without pain being felt at the time. |  |  |  |  |  |  |  |  |  |  |  |
| Controls |  |  |  |  |  |  |  |  |  |  |  |  | Remedial Actions |  |  |  |  |  |  |  |  |  |  |  |
| - Wear impermeable protective clothing to ensure that cement or any cement/water mix does NOT come in contact with the skin. - Ensure that wet concrete does NOT enter the boots and that people do NOT kneel on wet concrete. Wear impermeable gloves. - Wherever there is a risk of cement powder or cement/water mixtures entering the eye, wear dust proof goggles. Do not eat, drink, or smoke when using. Wash hands thoroughly after use. - Ensure good ventilation or ensure suitable RPE is worn. - When laying concrete, impermeable gloves, boots, and trousers will be necessary. General awareness talks on delayed effects. |  |  |  |  |  |  |  |  |  |  |  |  | - Do NOT wash cement down sewers, drains, watercourses. - Recover the spillage in a dry state if possible. - Use clean-up methods that do NOT cause airborne dispersion. Use wet mop/vacuum. - Collect wet cement and place in container. - Ensure all cement has hardened before disposal. |  |  |  |  |  |  |  |  |  |  |  |
| Disposal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous Waste? ☐ |  |  |  |  | Skip ☒ |  |  |  |  |  |  |  | Other ☒ Do NOT use after use by date. Do NOT flush into sewers, drains, watercourses. Dispose or recycle according to local authority |  |  |  |  |  |  |  |  |  |  |  |
| Return to Depot ☐ |  |  |  |  | Return to Supplier ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FIRST AID |  |  |  |  |  |  |  |  |  |  |  |  | FIRE FIGHTING |  |  |  |  |  |  |  |  |  |  |  |
| Move person to fresh air. If discomfort persist, seek medical attention. Remove any contaminated cloths. Wash off with plenty of water, seek medical attention if irritation continues. |  |  |  |  | Do NOT rub eyes. Remove any contact lenses and flush with water. (Use isotonic water if able to.) - Seek medical attention if irritation continues. If ingested do NOT induce vomiting. Wash out mouth with plenty of water, seek medical advice if irritation persists. |  |  |  |  |  |  |  | WATER ☒ |  | FOAM ☒ |  | POWDER ☒ |  |  |  |  | CO2 ☒ |  | CHEMICAL ☒ |
|  |  |  |  |  |  |  |  |  |  |  |  |  | PRECAUTIONS Use extinguishing method best suited for surrounding materials. |  |  |  |  |  |  |  |  |  |  |  |

| SITE SPECIFIC CONTROLS (TO BE COMPLETED ON SITE) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Work Activities Being Undertaken |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exposure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Persons |  |  |  | Frequency |  |  |  |  | Duration |  |  |  |  | Any specific training requirements? |  |  |  |
| 5 -10 |  |  |  | Daily |  |  |  |  | 8 Hours |  |  |  |  | ☐ |  |  |  |
| Personal Protective Equipment (PPE) Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | ☐ |  | ☒ |  | ☒ |  |  | ☐ |  | ☐ |  | ☒ |  |  | ☒ |  | ☐ |
| Eye Protection | Ear Protection |  | Hard Hat |  | Footwear |  |  | Overalls |  | Visor |  | Gloves |  |  | Face Mask |  | Respirator |
| Storage Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCKED CAGE |  | BUNDED CONTAINER |  |  |  |  | PROTECTION FROM WEATHER |  |  |  | COSHH STORAGE ROOM |  |  |  |  | SEALED CONTAINER |  |
| ☐ |  | ☐ |  |  |  |  | ☐ |  |  |  | ☐ |  |  |  |  | ☐ |  |
| Waste Management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SEGREGATED STORAGE |  |  |  |  |  | SEALED CONTAINERS |  |  |  |  |  |  | WASTE CARRIER DISPOSAL / RECYCLING LICENCE |  |  |  |  |
| ☒ |  |  |  |  |  | ☒ |  |  |  |  |  |  | ☐ |  |  |  |  |
| Risk Rating Following Control Measures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOW |  |  |  |  |  | MEDIUM |  |  |  |  |  |  | HIGH |  |  |  |  |
| ☒ |  |  |  |  |  | ☐ |  |  |  |  |  |  | ☐ |  |  |  |  |

| Site Specific RA Prepared by: | Si King | Position: | Contracts Director |
| --- | --- | --- | --- |
|  |  | Review Date: | 28/09/2023 |