December 2023

Cover system validation report overview document, 31A North Street, Emsworth, PO10 7DA

Sirs

Parchow Groundworks have been advised that there is some confusion that may have been inadvertently caused by the report that we’d previously issued.

The purpose of this overview document is to explain the installation of the cover system in terms sequential process, and where appropriate, explain any deviation from the cover system noted with in the GCC RMS, albeit the change being one of an enhanced performance.

To aid understanding, the overview will deal with the bullet points noted by the HBC Environmental Control officer and then proceed to describe the works process to better understand the installation of the cover system and to enable all stakeholders to have adequate comfort in its efficacy and suitability for purpose.

Garden areas

The total garden areas given are accurate and indicate all areas that are subject to the specification shown in 4.3.2 of the GCC report indicating this. This area is 190.53m2. This covers the areas within the gardens inclusive of paths and patio’s etc

Conveyance proof

Page 8 of the previously submitted report may not have made clearly enough that there were in additional to the 2 TJ Transport Ltd topsoil loads an additional 3 delivered by Emmerson Transport Ltd.

However, in addition to this there were a further 2400l’s of topsoil delivered from a merchant  and  a bulk delivery from Eurogreen Transport ltd from the same as the TJ & Emmerson deliveries that were not included.

Apologies, these are now appended to the overview document.

The 600mm cover system required 114.32m3 of topsoil volume and the deliveries noted, which equates to 6.35 loads, provide the required volume to comply with the RMS cover system design.

Retained made soils on site.

All materials noted in the images are in areas that are hard landscaped and not subject to the requirements of the cover system, as noted at the foot of 4.2 in the GCC report.

Image 1 in the report is a perfect example of where all material within these areas were removed and replaced with BS3882 topsoil to the full section depth of 600mm within the area that the report targets for RMS cover system.

Works sequence

In order to mitigate against possible cross contamination, the works were broadly segregated between the North & East blocks so as to make them manageable and enable material to be tracked accurately on site.

The East block was blocks were undertaken first, commencing 5/6/23. The unit’s green space areas were plotted by our setting out engineer and an area set aside between the 2 blocks with a sacrificial membrane placed to contain the made material that was being excavated prior to export from site.

The design formation level was 600mm below FGL, with discussions having taken place between PWP and HBC over the replacement of the biotic layer and the subsoil, with the replacement of both with BS3882 topsoil. The reasons for the change were manyfold but primarily that commercially available quantities of BS8601:2013 are very limited locally and as such, while not attractive for cost, replacing with the accredited topsoil assured us of the source characteristics that the GCC report required.

Additionally, the change from 3 materials in the strata make-up to 1 simplified the process and thereby made errors of depth and contaminated layers far less likely.

Once at the formation level the site operatives placed Terram H-Viz anti-dig membrane [data sheet appended] in the area, ensuring that this abutted the hard landscaped areas sufficiently to alert anyone excavating in the future to the possible risks.

Upon completion of this stage of the process the BS3882 Topsoil was placed [between 12/6/23-15/6/23] directly in the excavation rather than placement elsewhere onsite, again to limit any possibility of cross-contamination.

The North block works commenced 15/6/23 with the same process undertaken regards using the proposed hard landscaped area between the blocks to temporarily store the export material away from the excavation. Again, formation level was established at 600mm below FGL and the Terram ani-dig barrier placed prior to the BS3882 topsoil being placed directly into the excavation.

Appendix

- Merchant invoices for bagged topsoil

- Terram data sheet

- Haulier conveyance information for topsoil that was previously noted sent