HAVANT BOROUGH COUNCIL

| APPLICATION No. | APP/23/00671 |
| --- | --- |
| LOCATION | 8 Northlands Park, Emsworth, PO10 7EY |
| PROPOSAL | Construction of 6.3m high boat shed in garden. |
| RECOMMENDATION | No objection, subject to conditions |
| DATE | 5th October 2023 |

Reasons for Recommendation

The Council seeks to protect tree coverage in the Borough, sympathetically incorporating existing features into the overall design of the scheme including measures taken to ensure their continued survival.

The application is for the construction of a detached shed to accommodate a boat and water sports equipment, but to allow maintenance work to the boat the shed is unusually tall. The location of the shed is to be in the eastern side garden, which has been recently cleared of 24 trees subject to TPO 1838 by consent in application APP/21/01060.

The removal of the trees opened up the site, but any potential development has to take into consideration the surrounding trees. Following pre-application advice, the applicants has commissioned a Tree Survey and Arboricultural Impact Assessment (AIA).

The AIA outlines the potential impact of the development on the surrounding trees, the method of works to minimise harm to the trees and tree protection measures to be employed pre, post and during construction, in accordance with BS5837:2012. The AIA identifies the need to crown lift T18 and the need to construct the shed above ground level to comply with Flood Risk Assessment requirements, which also benefits the trees by minimising disturbance to tree roots.

The AIA also stipulates the need for measures to contain cement leachate during the construction of the foundations and that no underground services (including soak-aways) shall be located in any part of the Construction Exclusion Zones or Restricted Activity Zones. Although the development does extend into the Root Protection Area (RPA) of surrounding trees, it would only impact the edge of the RPAs and this would not have a significant detrimental impact on the surrounding trees.

Based on the information submitted, if the officer was minded to grant permission the potential impact of the surrounding trees should be secured by way of condition, and would be in accordance with Policy CS16 and DM8 of the Havant Core Strategy (2011).

Recommendation:

Approval subject to the following conditions;

- Pruning works:

Any required tree works shall be pruned in accordance with the with BS5837:2012: Tree Survey and Arboricultural Impact Assessment  (23_5837_07_36) provided by ROAVR Group and the recommendations in British Standard BS3998:2010 (Recommendations for Tree work).

Reason:  To ensure the continuity of amenity value afforded by the trees in question and having due regard to policies CS16 and DM8 of the Havant Borough Core Strategy (2011) and the National Planning Policy Framework.

- Tree protection measures:

The development hereby permitted shall only be carried out in accordance with BS5837:2012: Tree Survey and Arboricultural Impact Assessment  (23_5837_07_36) provided by ROAVR Group.

Reason: To ensure the continuity of amenity value afforded by the trees in question and in accordance with policies CS16 and DM8 of the Havant Borough Core Strategy (2011) and the National Planning Policy Framework.