Principal Conservation Officer – No Objection

57 the High Street is a Grade II listed building which dates from the late 18th century. The building is of a symmetrical appearance and has three storeys with ground floor shop fronts. It is also located within the Emsworth Conservation Area and forms an important visual stop at the west end of Queen Street.

Section 16 of the Planning (Listed Buildings and Conservation Areas) Act 1990 ("1990 Act") requires local planning authorities to have special regard to the desirability of preserving a listed building, its setting or any features of special architectural or historic interest when considering whether to grant listed building consent.

I believe the design of the proposals would improve the current appearance of the building to the rear. The zinc roof is a superior quality material to the current concrete roofing panels and the proposed rooflights would sit comfortably within the roofscape. The rooflights will need to be of a conservation style with a condition attached requesting details to be submitted.

Condition:

- Rooflight details to be submitted prior to development

- Window details to be submitted (existing rear windows are to be replace with new black aluminium crittall windows