Bunting Gardens - Schedule of Proposed External Materials

| Location | Material |
| --- | --- |
| New footpaths (Highways) | Asphalt concrete paving in accordance with with HCC Specification (Hampshire County Council Model Documents), The National Specification for Highways Works (SHW) Series 800 and BS EN 13108-1 |
| Parking Areas (Highways) | Asphalt concrete paving in accordance with with HCC Specification (Hampshire County Council Model Documents), The National Specification for Highways Works (SHW) Series 800 and BS EN 13108-1 |
| Tactile Paving (Highways) | Concrete Tactile Flag Paving - Tobermore Corduroy, colour; buff. |
| Kerbs and Edgings (Highways) | Standard precast concrete kerbs and edgings to BS EN 1340 |
| Road markings to demarcate parking bays (Highways) | White thermoplastic paint system to Road Safety Markings Association standard specification document for road marking and road studs (StanSpec). |
| Footpaths, front gardens and patios (non-highway) | Concrete Flag Paving - Tobermore Textured Paving Slabs, colour: natural. With concrete sett feature elements - Tobermore Pedesta, colour; natural. |
| Walls (garden walls and house walls) | Brickwork - Ibstock Coleridge Yellow Multi |
| Railings and guardings to front gardens | Galvanised steel to BS EN ISO 1461 and powder coated to BS 1722-16, colour; black |
| Windows & Patio Doors | Composite aluminium clad timber windows - Nordan NTech Villa, colour; Window Grey RAL 7040 |
| Window Cills | Cast concrete cills - Forticrete Stooled Ciff Ref T1-40, colour; Portland |
| Front Doors | Nordan HomeGuard, colour Window Grey RAL 7040 |
| Roof | Concrete interlocking tiles - Marley Duo Edgemere, colour; Smooth Brown |
| Rainwater Goods | Aluminium box gutters and rainwater pipes - Dales Delta 200 & 150, with Square Contemporary rainwater pipes, colour; RAL 7040 |
| Fencing | Pressure treated softwood close-boarded fences |