Arboricultural Method Statement

& Tree Protection Plan for:

Plan in accordance with BS 5837:2012

| Client | Paul Barber |
| --- | --- |
| Location | The Willows |
| Date of site visit | 10/5/16 |
| Report prepared by | Joe Jackman |

Contents:

Introduction 	2

Sequence of events 	3

Arboricultural Supervision 	3

Root Protection Areas 	4

Restrictions within Tree Protection Areas 	6

Tree Protection Fencing 	6

Method Statement for Hand Digging	6

Site Notes 	7

Introduction

Jackman Arb have been instructed by Paul Barber to carry out a tree report with root protection plan at the Willows, Lower road.

This Arboricultural Method Statement has been complied to ensure the safe and healthy retention of all trees on and around the site. Integral to achieving this goal is the implementation of the special construction details and protection methods detailed within this report.

This Method Statement must be made available to all contractors and operatives on site during the construction process so that they fully understand the importance of the measures set out for tree protection.

For details of tree locations and types of special protection methods, reference should be made to the Tree Protection Plan.

It should be noted that this is a site-specific Arboricultural Method Statement produced solely for the physical protection of those trees identified within the above plan and is not relevant to any other site or situation.

This report has been compiled from data achieved by Visual Tree Assessment and no more detailed survey has been carried out.

Sequence of events

The following sequences are governed by operational constraints and subject to change. The developer’s arboriculturalist must be noted of any changes to this schedule:

Pre-development Stage

- This method statement to be approved in writing by the Local Planning Authority.

- Pre-commencement site meeting between the Tree Officer, Arboricultural Consultant and Client.

- This meeting must take place before any development to confirm the protection of trees on and adjacent to the site in accordance with the Method Statement.

- Tree protection measures installed to trees.

- Site to be inspected by arboriculturalist.

Development Stage

- This stage is subject to site monitoring visits by the client’s arboriculturalist at intervals as agreed at the pre-commencement site meeting. These visits are to ensure that the agreed protection measures are functional and correctly achieving their purpose.

- Site accessible to construction traffic.

Post Development

- Removal of Protective Fencing as agreed by the developer’s arboriculturalist.

- An arboricultural supervision statement, the contents of which are to be agreed at the pre-commencement meeting, is submitted to and approved in writing by the Local Planning Authority on completion of development to discharge condition 8 of the planning permission.

Arbroicultural Supervision

Arboricultural supervision is to be carried out at all crucial stages throughout the development

process to ensure detailed tasks are carried out as per the approved methodology. At points as

detailed in the above sequence of events and during:

- The work to extend the driveway that takes place within RPA’s.

This supervision will require the arboriculturalist to be present throughout the tasks, to ensure all the arboricultural objectives are met.

If the task is to take a long period of time, provided the arboriculturalist is satisfied, the supervision may be reduced to telephone contact between the site Project Manager and the arboriculturalist.

The local authority arboriculturalist will have free access to the site and pass any recommendations direct to the client’s arboriculturalist.

Any alterations to the Protective Fencing should be approved by the client’s arboriculturalist and Local Authority arboriculturalist.

Root Protection Area

The following table details the tree survey data and based on this, root protection areas (RPA’s) have been determined for every retained tree. The RPA’s are designed to protect at least a functional minimum of tree root mass in order to ensure that the trees survive the construction process.

It is the responsibility of everyone engaged in the construction process to respect the tree protection measures and observe the necessary precautions within and adjacent to them.

| Tree/Group No. | Species | Height (ft) | Stem Diameter (cm) | Single/multistemmd (S/M) | Crown spread | Condition (G, M, P) | Comments, Structural condition | Reccomendations | RPA | Age Class (Y,EM,M,LM,V) | Category | Hours |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Sycamore | 50 | 62 | S | 22 | M | Next to wall. Sizeable deadwood in crown. Deadwood predominantly leaning towards Lower Rd, cavity at 13ft. Previous storm damage on main trunk. | Remove deadwood | RPA: 174m2 Radius: 7.4m | M | B | 1 |
| 2 | Sycamore | 65 | 95 | S | 37 | G | Larger tree in group. Ivy on main stem. Two main leading stems at 20ft, broad crown. Conjoined branches at 30ft-signs of stress- branches rubbing. Possible debris build up. | Remove Ivy Remove deadwood Inspect unions | RPA: 408m2 Radius: 11.4m | M | B | 2 |
| 3 | Ash | 30 | 25 | S | 12 | M-P | Small tree in group. Ivy on main stem. Supressed by trees no. 2 and 4. Phone line under tree | Fell | RPA: 28m2 Radius: 3m | Y | C | 1 |
| 4 | Ash | 60 | 60 | S | 35 | G | Slight lean with three main stems. Breaking out at 13ft. next to wall growing over road. Healthy tree. Small ivy on lower trunk. Decay at base of trunk | Remove deadwood. Raise drooping branches over road by 20% | RPA: 163m2 Radius: 7.2m | M | B | 2 |
| 5 | Ash | 58 | 57 | S | 35 | M-P | Heavily weighted N side. Suppressed by adjacent tree. Large tear at 10ft on main trunk. Over phone cable | Fell | RPA: 147m2 Radius: 6.8m | E-M | C | 4 |
| 6 | Laurel | 22 | 30 | M | 25 | M | Large Laurel. Deadwood | Prune away from entrance | RPA: 28.3m2 Radius: 3m | MM | B | 1 |
| 7 | Laurel | 23 | 22 | S | 15 | M | Laurel next to fenceline | Remove deadwood and raise from the fence | RPA: 40.7m2 Radius: 3.6 | M | B |  |
| 8 | Bay | 31 | 30 | M | 20 | M | Large bay tree heavily weighted S | Reduce by 25% | RPA: 21.9m2 Radius: 2.6m | M | B | 1 |
| 9 | Laurel | 25 | 21 | S | 20 | M | Resting on fence | Prune off fence | RPA: 20m2 Radius: 2.5m | M | C |  |
| 10 | Laurel | 25 | 21 | S | 20 | M | Resting on fence | Prune off fence | RPA: 20m2 Radius: 2.5 m | M | C |  |

Restrictions within Tree Protection Areas

Inside the exclusion area of the Protective Fencing, the following shall apply:

- No mechanical excavation

- No excavation by any other means without arboricultural site supervision.

- Hand digging must follow the method statement set out below

- No ground level changes whatsoever.

- No storage of plant or materials.

- No storage or handling of any chemicals.

Tree Protection Fencing

Prior to any work commencing protective (herras) or similar fencing 2.5 Metres in height will need to be installed based on the RPA detailed above. Exact locations of protective fencing to be agreed at the pre-commencement site meeting.

Method Statement for Hand Digging

In order to reduce any significant damage to tree roots within the RPA all soil stripping and excavation must be carried out by hand digging only. Where construction is within the RPA the use of specialist engineering solutions may be required.

- Unless instructed otherwise, all hand digging within the RPA must be under the supervision of the client’s arboriculturalist.

- All tree roots uncovered must be wrapped immediately in dry sacking until backfilling takes place. Tree roots must not remain exposed.

- Clumps of fibrous roots must not be severed and need to be retained as in above.

- Severance of structural roots 25mm or more in diameter is not permitted without prior permission of the client’s arboriculturalist.

- If roots are accidentally damaged the tree owner must be notified and given the opportunity to inspect the damage before it is covered over.

Site Notes

If vehicle access is needed within the RPA, then suitable measures should be in place, such  as padded matting, or raised/ bridging platform, reducing any impact on RPA