Twittens wall support & protection

Application app/14/00065

Condition 2

Grove road havant

Method statements accompanying drawing

- Three in number 225 x 50 mm timber bearers are to be secured to the inside face of twittens wall as per drawings

- Scaffold tubing is to be braced against the bearers and cantilevered into cast concrete padstones set into the ground

- Additional tubing is to be cross braced and attached to the diagonal bracing

- Once construction begins on the dwelling the eastern elevation rat-trap wall to be taken down and raked back leaving a 45degree portion of the wall in tacked providing additional support for the twittens wall

- A same approach to the post 1974 curved wall to the western elevation is to be adopted during the construction process

- Once the archeological proposal has been agreed and implemented herris panels will be clipped inside the wall bracing giving a 100mm void between the twittens wall and the protection .