14 bath Road, Emsworth, PO10 7EP

Conservation Officer  No objection

14 Bath Road is an end of terrace property of brick and render finish to the first floor, with a slate roof. To the front of the property there is a two storey bay and a porch with a pitched roof. The property is separated from the next terrace by a shared access.

The property sits within Town Mill Pond (Bridgefoot path/Bath Road character sub area of the Emsworth Conservation Area. The Conservation Area boundary runs along the bottom of the gardens of Bath Road. The property over looks Mill Pond and allotments to the rear.

The proposal is to rearrange the internal lay out of the house and replace the existing conservatory with a new extension covering a similar footprint with a balcony above. There are a number of changes to the windows.

These comments address the impact on the Emsworth conservation Area and not planning issues.

Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires planning authorities, when considering whether to grant planning permission for development which affects a Conservation Area, to have special regard to the desirability of preserving or enhancing the character and appearance of that area.

The significant view of the property from within the Conservation Area is of the front elevation from the waters edge/across the water.  The Emsworth Conservation Appraisal identifies the fronts of the properties in Bath Road as being important. The only change to the front elevation is the rising of the sill height of the window above the front door. Whilst the properties in Bath Road all have slight variances to their front elevation through different glazing arrangements the window at first floor are on the whole of uniform size along the road. Considering the variation in porches and dormers to the front elevation of neighbouring properties, the raising of the sill height is not deemed to have a significant impact on the Conservation Area.

The changes to the rear are more marked. The replacement of the conservatory with two sets of bi-fold doors and a tiled roof reflects the extension to the adjoining property. The decision to replace the windows and doors with white UVPC to match is more in keeping with the front of the property and neighbours. These changes to the ground floor are obscured from view from the allotments by shrubbery. However the first floor is more prominent. The properties in Bath Road have a fairly uniform window pattern and size at first floor. This application departs from this arrangement with a proposal of four bi-fold doors across the balcony and single pane window. This is deemed not to be harmful to the character of the Conservation Area as it is on the rear elevation and not a key view, although visible from the allotments it is less prominent than the dormer windows visible on other properties.