Habitats Regulations Assessment (HRA)

Screening and Appropriate Assessment (AA)

PLEASE NOTE:  Undertaking the HRA process is the responsibility of Havant Borough Council as the Competent Authority for the purpose of the Habitats Regulations. However, it is the responsibility of the applicant to provide the Competent Authority with the information that they require for this purpose.

| HRA drafting date: | 02/10/2025 |  |
| --- | --- | --- |
| HRA completion date: |  |  |
| Application reference: | APP/25/00501 |  |
| Application address: | 4 Wittering Road, Hayling Island, PO11 9SP |  |
| Application description: | Extensions and alterations to all elevations including the installation of 1No balcony |  |
| Lead Planning Officer: | Selina Donophy |  |
| Please note that all references in this assessment to the ‘Habitats Regulations’ refer to The Conservation of Habitats and Species Regulations 2017 (as amended). |  |  |
| Stage 1 - details of the plan or project |  |  |
| European site potentially impacted by planning application, plan or project: |  | - Chichester & Langstone Harbours Special Protection Area (SPA) - Chichester & Langstone Harbours Ramsar site - Solent Maritime Special Area of Conservation (SAC) |
| Is the planning application directly connected with or necessary to the management of the site (if yes, Applicant should have provided details)? |  | No. The development consists of alterations to an existing dwelling. There will be no increase in the number of residential dwellings. The development is neither connected to nor necessary to the management of any European site. |
| Are there any other projects or plans that together with the planning application being assessed could affect the site? |  | None in relation to this site. |
| Stage 2 - HRA screening assessment |  |  |
| Screening under Regulation 63(1)(a) of the Habitats Regulations – The Applicant to provide evidence so that a judgement can be made as to whether there could be any potential significant impacts of the development on the integrity of the SPA/SAC/Ramsar. |  |  |
| Extensions and alterations to all elevations including the installation of 1No balcony There is considered to be potential for the construction works to result in impacts to the Solent Maritime SAC and Chichester & Langstone Harbours SPA and Ramsar site. In summary: Potential for construction impacts. Potential impact pathways include noise and visual disturbance (affecting overwintering bird species), and airborne and waterborne pollution (affecting the water environment and coastal habitats). Would the proposal lead to a likely significant effect on European site integrity? Yes, due to potential direct impacts on the SAC/SPA/Ramsar habitat/species. (If yes, continue to Stage 3). |  |  |
| Stage 3 - Appropriate Assessment |  |  |
| Appropriate Assessment under Regulation 63(1) - if there are any potential significant impacts, the applicant must provide evidence showing avoidance and/or mitigation measures to allow an Assessment to be made. The Applicant must also provide details which demonstrate any long-term management, maintenance and funding of any solution. |  |  |
| The application is accompanied by an Extended Phase 1 Ecological Assessment and Bat Roost Assessment (Phillips Ecology, May 2022): This identifies the following avoidance / or mitigation methods: Designated sites In order to ensure that adverse effects on the identified designated sites are avoided, the following measures are proposed during the property construction phase: A suitable temporary visual screen will be installed along the eastern boundary of the site between 1st October and the 31st March. This could be Heras fencing with dust netting or fencing panels. This will ensure that construction works are screened Any temporary site compound set out for the storage of equipment, materials or machinery and cutting or mixing materials associated with the construction works should be located as far from the designated sites as possible and not within 10m. All stockpiles of loose material should be covered to prevent run-off during periods of rain or wind dispersal of dust. Any temporary compound and all associated construction materials will be removed on completion of the works. The key factor in safeguarding against water pollution during the construction phase will be the diligence of the site manager and operatives. Therefore, the site manager and all site operatives should be made aware of the presence of the Chichester and Langstone Harbours and Solent Maritime and should understand the requirement to ensure that construction works do not encroach into the designated sites. f possible, store fuel and oils off site. If this is not possible then a proprietary bunded storage facility of the smallest practicable size located as far from the designated sites as possible and in all cases at least 10m. If possible, undertake refuelling off site. If this is not possible refuelling should take place as far from the designates sites as possible on an impermeable surface. In practice this is likely to comprise a hardstand portion of the driveway. A spill kit must be maintained on site. Any construction lighting must be directed away from the designated sites at all times. The Council will secure these measures by the imposition of appropriate Construction Environmental Management Plan conditions. On this basis it is concluded that the proposed development would not result in a likely significant effect on European designated sites. |  |  |
| Stage 4 – Summary of the Appropriate Assessment (To be carried out by the Competent Authority (the local planning authority) in liaison with Natural England |  |  |
| This assessment represents Havant Borough Council’s Appropriate Assessment as a Competent Authority in accordance with requirements under Regulation 63 of The Conservation of Habitats and Species Regulations 2017 (as amended), Article 6 (3) of the Habitats Directive and having due regard to its duties under Section 40(1) of the NERC Act 2006 to the purpose of conserving biodiversity. Consideration of the Ramsar site/s is a matter of government policy set out in the National Planning Policy Framework. This assessment concludes that the application will have a likely significant effect in the absence of avoidance and mitigation measures on the above Solent European sites. The authority has concluded that the potential adverse effects arising from the proposal can be appropriately addressed by the imposition of conditions to secure the mitigation and enhancements proposed in the Phase 1 Ecological Assessment together with a Construction Environmental Management Plan. It can therefore be concluded that there will be no adverse effect on the integrity of the designated sites identified above. |  |  |
| Natural England Officer: |  |  |
| Summary of Natural England’s comments: |  |  |