Havant Borough Council

Date – 5/11/25

Officer – Rachael McMurray, Principal Heritage Officer

Application – APP/25/00491

Site Address – 25 Tower Street, Emsworth

Proposal:

Single storey side extension, changes to fenestration, alterations to existing front boundary to include new brick piers and gates, roof alterations to eastern elevation, and garage conversion.

Heritage Consultation Response

Site / Location

The application site is located on Tower Street, whereby the south elevation fronts directly onto Chichester Harbour, which is a designated National Landscape. The application site comprises a two-storey dwelling of modern construction, with an integral garage. The eastern roof slope features a catslide roof and three roof light windows. The east boundary of the site comprises a brick wall which forms the boundary with a public right of way which extends down on the harbour.

The site is also located within the Emsworth Conservation Area and the immediate neighbour to the west at No. 23 Tower Street, is a Grade II listed building.

Legislation

When assessing the proposal, the following legislation and policy context is taken in consideration.

Planning (Listed Building and Conservation Areas) Act 1990

Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires planning authorities, when considering whether to grant planning permission for development which affects a Conservation Area, to have special regard to the desirability of preserving or enhancing the character and appearance of that area.

National Planning Policy Framework (revised December 2024)

Chapter 16 of the NPPF relates to the Historic Environment including paragraphs 202-221. The following are most relevant:

Para 212 states that:

‘When considering the impact of a proposed development on the significance of a designated heritage asset, great weight should be given to the asset’s conservation (and the more important the asset, the greater the weight should be). This is irrespective of whether any potential harm amounts to substantial harm, total loss or less than substantial harm to its significance’.

Para 214 states that:

‘Where a development will result in substantial harm to, or total loss of, the significance of a designated heritage asset, permission should be refused, unless this harm is necessary to achieve substantial public benefits, or a number of criteria are met. Where less than substantial harm is identified paragraph 208 requires this harm to be weighed against the public benefits of the proposed development

Para 215 states that:

‘Where a development proposal will lead to less than substantial harm to the significance of a designated heritage asset, this harm should be weighed against the public benefits of the proposal including, where appropriate, securing its optimum viable use’.

The building subject to this application is not listed, but is deemed to be a non-designated heritage asset (as set out in the introduction) and therefore the following paragraph is also relevant:

Para 216:

‘The effect of an application on the significance of a non-designated heritage asset should be taken into account in determining the application. In weighing applications that directly or indirectly affect non-designated heritage assets, a balanced judgement will be required having regard to the scale of any harm or loss and the significance of the heritage asset’.

Havant Borough Local Plan (Core Strategy 2011 and Allocations Plan 2014)

Policy CS11 – Protecting and Enhancing the Special Environment and Heritage of the Havant Borough

Policy CS11 (4) states that planning permission will be granted for development which:

‘Protects and where appropriate enhances the borough’s statutory and non-statutory heritage designations by appropriately managing development in or adjacent to conservation areas, listed buildings, scheduled ancient monuments, historic parks and gardens, archaeological sites, buildings of local historic or architectural interest’.

Policy CS16 – High Quality Design

Policy CP16 (1a) of the Core Strategy states that:

‘Planning permission will be granted for development that is designed to a high standard, which helps to create places where people want to live, work and relax. All development should demonstrate that its design:

- Identifies and responds positively to existing features of natural, historic or local character within or close to the proposed development site’.

- Integrates with existing local landscape features, promotes wildlife and biodiversity and/or applies characteristics of the local area into the design of a scheme wherever possible to create variety and interest;

- Uses the characteristics of the locality to help inform the design of the new development including heights, massing, existing buildings lines, plot widths and depths, materials and proportions of windows and doors;

- Is well connected to and integrates with the immediate local area and the wider area by linking to existing pedestrian and cycle routes and encouraging people to use public transport where possible; and

- The development does not cause unacceptable harm to the amenity of neighbours through smell, the loss of privacy, outlook, noise and overlooking.

Policy DM20 (Allocations Plan) - Historic Assets

Policy DM20 states that;

‘Planning permission will be granted for development proposals that conserves and enhances the historic assets of Havant. It also states that application which affect, or have the potential to affect, heritage assets are expected to provide a Heritage Statement that:

1. Describe the significance of the asset and its setting, using appropriate expertise

and where necessary original survey, at a level of detail proportionate to its

significance and sufficient to understand the potential impact of the proposal.

2. Sets out the impact of the development on the heritage assets and mitigation that is

proportionate to the impact and the significance of the heritage asset, including

where possible positive opportunities to conserve and enjoy heritage assets.

The Emsworth Neighbourhood Plan – Heritage Policies

Assessment

The proposal includes the following works:

- A single storey side extension to the west elevation

- Changes to fenestration

- Alterations to the existing front boundary to include new brick piers and gates

- Roof extension to the eastern facing roof slope

- Garage conversion

The originally submitted plans included a large chimney on the east roof elevation however, this has since been removed from the proposal and amended plans submitted to reflect this change.

With regard to the proposed works to the main house, these comprise a single-storey extension to the west elevation, the conversion of the existing integral garage to provide additional living accommodation, and a modest build-up of the eastern roof slope to create further space at first-floor level. Both extensions would be constructed using materials that match the existing dwelling, ensuring visual continuity.

The single-storey extension would be subservient in scale, featuring a pitched roof designed to reflect the character and form of the existing property. The proposed alteration to the eastern roof slope would result in a slight increase in height; however, this change is considered modest and would largely maintain the existing roof pitch and profile. New windows would match the existing white uPVC units currently in place.

The single-storey extension is not considered to cause harm to the significance of the Emsworth Conservation Area. While the minor roof build-up would slightly reduce the existing visual gap at the end of the principal view extending from Tower Street to the harbour frontage, the change is limited in extent. The sense of openness at roof level would be sufficiently retained so as not to adversely affect this aspect of the conservation area’s character

With regards to the proposed boundary alterations, this would include a new pair of vehicular gates and the creation of a new pedestrian gate. This would be within the modern part of the boundary wall rather than the flint section and therefore, no loss of historic fabric would occur. The proposed gates are considered to be suitable in terms of their design (timber effect finish gates) and matching brick pier and would not result in harm to the significance of the conservation area.

Overall, it is considered that the proposals would not result in harm to the Emsworth CA and therefore meet the test set out in S72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 ("1990 Act"), as well as being compliant with the relevant policies in the NPPF and Local Plan.

Conclusion

NO OBJECTION subject to the following condition:

- Unless materials are expressly specified as matching those of the existing dwelling on the approved plans, details and / or samples of all new external materials to be used in the construction of the development hereby permitted shall be submitted to and approved in writing by the Local Planning Authority prior to their use on site. The development shall thereafter be carried out in full accordance with the approved details.

Reason: To ensure a satisfactory appearance to the development and to preserve the character and appearance of the Emsworth Conservation Area.