Havant Borough Council

Date – 28/11/2025

Officer – Rachael McMurray, Principal Heritage Officer

Application – APP/25/00590

Site Address – 18 East Street, Havant

Proposal – Conversion of ground floor into a care home facility for 6 residents, incorporating some alterations at rear ground floor level, with 3No. supported living flats at first floor.

Heritage Consultation Response

Site / Location

The application site is located on East Street in Havant town centre and within the St Faiths Conservation Area. It comprises a vacant, two-storey building, formerly Barclays Bank, which has been extensively extended at the rear. To the rear of the building is a private car park which is accessed from Twittens Way and enclosed by a 2m high brick wall.

The building dates from the mid nineteenth century and contributes positively, together with other historic buildings in this part of East Street, to the character and significance of the St Faiths Conservation Area. Due to its historical and architectural interest, the building is deemed to be a non-designated heritage asset (NDHA).

Legislation

When assessing the proposal, the following legislation and policy context is taken in consideration.

National Planning Policy Framework (revised December 2024)

Section 72 of the Planning (Listed Buildings and Conservation Areas) Act 1990 requires planning authorities, when considering whether to grant planning permission for development which affects a Conservation Area, to have special regard to the desirability of preserving or enhancing the character and appearance of that area.

Chapter 16 of the NPPF relates to the Historic Environment including paragraphs 202 – 221. The building subject to this application is not listed but is deemed to be a non-designated heritage asset (as set out in the introduction) and therefore the following paragraph (216) is most relevant to the determination of this application:

‘The effect of an application on the significance of a non-designated heritage asset should be taken into account in determining the application. In weighing applications that directly or indirectly affect non-designated heritage assets, a balanced judgement will be required having regard to the scale of any harm or loss and the significance of the heritage asset’.

Havant Borough Local Plan (Core Strategy 2011 and Allocations Plan 2014)

Policy CS11 – Protecting and Enhancing the Special Environment and Heritage of the Havant Borough

Policy CS11 (4) states that planning permission will be granted for development which:

‘Protects and where appropriate enhances the borough’s statutory and non-statutory heritage designations by appropriately managing development in or adjacent to conservation areas, listed buildings, scheduled ancient monuments, historic parks and gardens, archaeological sites, buildings of local historic or architectural interest’.

Policy CS16 – High Quality Design

Policy CP16 (1a) of the Core Strategy states that:

‘Planning permission will be granted for development that is designed to a high standard, which helps to create places where people want to live, work and relax. All development should demonstrate that its design:

- Identifies and responds positively to existing features of natural, historic or local character within or close to the proposed development site’.

- Integrates with existing local landscape features, promotes wildlife and biodiversity and/or applies characteristics of the local area into the design of a scheme wherever possible to create variety and interest;

- Uses the characteristics of the locality to help inform the design of the new development including heights, massing, existing buildings lines, plot widths and depths, materials and proportions of windows and doors;

- Is well connected to and integrates with the immediate local area and the wider area by linking to existing pedestrian and cycle routes and encouraging people to use public transport where possible; and

- The development does not cause unacceptable harm to the amenity of neighbours through smell, the loss of privacy, outlook, noise and overlooking.

Policy DM20 (Allocations Plan) - Historic Assets

Policy DM20 states that;

‘Planning permission will be granted for development proposals that conserves and enhances the historic assets of Havant. It also states that application which affect, or have the potential to affect, heritage assets are expected to provide a Heritage Statement that:

1. Describe the significance of the asset and its setting, using appropriate expertise

and where necessary original survey, at a level of detail proportionate to its

significance and sufficient to understand the potential impact of the proposal.

2. Sets out the impact of the development on the heritage assets and mitigation that is

proportionate to the impact and the significance of the heritage asset, including

where possible positive opportunities to conserve and enjoy heritage assets.

Assessment

The proposal is for the conversion of the ground floor into a care home facility for 6no. residents, which would include some alterations to the ground floor layout and conversion of the first floor to a further 3no. supported living at flats on the first floor. The proposal also includes renovation of the rear external space to include parking spaces and landscaped garden areas for the residents.  The proposal also includes an external fire escape on the rear of the building from first floor to ground floor.

In terms of its historic interest, the building represents a late Georgian/early Victorian, two-storey building with classical proportions, wooden sash windows, a clay tile roof, a rendered façade, corner brick quoin detailing and a traditional shopfront-style ground floor, including pilasters and a central fanlit doorway. The front elevation is deemed to be of the highest architectural and historic significance. The side elevations of the building are not rendered and display the original red brick facades. These elevations also contain timber framed windows. The rear elevation has been heavily altered by several modern extensions to facilitate the former bank use.

The proposed change of use does not include any alterations to the front, or side facades of the original building. There will be some minimal alterations to the rear and the proposal does include a rear fire escape, which will be attached to the building. It is also proposed to relocate the existing vehicular entrance into the car park further along the existing boundary wall. This wall is not deemed to be of historic interest and therefore this alteration would not result in harm to the conservation area.

Overall, given that the proposed works would not require any physical alterations to the external façades of the main historic building and that the works to the rear are minimum and in keeping with the building, it is considered that no harm would arise to either the NHDA, nor the St Faiths CA.

Conclusion

The proposal would not result in harm to the NDHA, nor the significance of the St Faiths CA and is therefore in accordance with Section 72 of the Planning (Listed Buildings and Conservation Areas) Act, along with the relevant sections of the NPPF and the adopted local plan.

NO OBJECTION subject to the following conditions:

- Details of all new external materials (including new windows / doors) to provided and agreed prior to use.