DEMOLITION ENVIRONMENTAL MANAGEMENT PLAN

Office Building, Southleigh Park, Eastleigh Road, Havant PO9 2PE

Figure 1 – Southleigh Park: building to be demolished highlighted green; Southleigh Park House (Grade II) adjacent

| Project Number and Title: | T2636 – Office Building, Southleigh Park, Havant |
| --- | --- |
| Document No.: | T2636-DEMP-01 |
| Revision No.: | 00 |
| Issue Date: | 07/10/2026 |
| Client: | Bargate Homes Ltd |
| Status: | Tender / planning issue. Read with Demolition Method Statement T2636 Rev 00 (24/08/2026). |

Document Control

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared by | Ian Martin — Estimating |  | 07/10/2026 |
| Reviewed by | Gerry Marchant — HSE Advisor |  |  |
| Approved by | Micky Watkins — TBC |  |  |

| Rev | Date | Description |
| --- | --- | --- |
| 00 | 07/10/2026 | First issue — to accompany the Demolition Method Statement in support of Bargate Homes’ planning submission and tender. |

Contents

1. Introduction and Purpose

2. Project and Site Details

3. Roles and Responsibilities

4. Environmental Aspects and Sensitive Receptors

5. Protection of Listed Structures

6. Noise Management

7. Vibration Management

8. Dust and Air Quality Management

9. Waste and Materials Management

10. Pollution Prevention, Drainage and Ground

11. Trees and Ecology

12. Traffic, Highways and Logistics

13. Community and Stakeholder Liaison

14. Monitoring, Inspection and Records

15. Environmental Incidents and Emergency Response

16. Training and Communication

Appendix A – Pre-commencement Confirmations

1. Introduction and Purpose

This Demolition Environmental Management Plan (DEMP) sets out how R Collard Ltd (RCL) will manage the environmental effects of demolishing the two-storey office building at Southleigh Park, Eastleigh Road, Havant, on behalf of Bargate Homes Ltd. It is written to align with, and should be read together with, RCL’s Demolition Method Statement T2636 Rev 00, which sets out the sequence of works.

The building sits within the grounds of Southleigh Park House, a Grade II listed mid-19th century country house (National Heritage List entry 1303673). Protection of the listed structures is therefore the controlling environmental and heritage issue for these works, and Section 5 sets out the specific measures RCL will apply. The DEMP also covers noise, vibration, dust, waste, pollution prevention, trees and ecology, traffic and community liaison.

The commitments in this DEMP form part of RCL’s site rules. They will be briefed at induction, carried into the task-specific RAMS, and checked daily by the RCL Site Manager.

1.1 Legislation and guidance

- Planning (Listed Buildings and Conservation Areas) Act 1990.

- Control of Pollution Act 1974 (Sections 60, 61 and 72 — Best Practicable Means) and Environmental Protection Act 1990 (Part III statutory nuisance; Section 34 Duty of Care).

- BS 5228-1:2009+A1:2014 (noise) and BS 5228-2:2009+A1:2014 (vibration) — Code of practice for noise and vibration control on construction and open sites.

- BS 7385-2:1993 and DIN 4150-3:2016 — building vibration damage criteria.

- IAQM Guidance on the Assessment of Dust from Demolition and Construction, v2.2 (January 2024).

- BS 6187:2011 Code of practice for full and partial demolition; Construction (Design and Management) Regulations 2015.

- Environmental Permitting (England and Wales) Regulations 2016; Waste (England and Wales) Regulations 2011; Hazardous Waste (England and Wales) Regulations 2005.

- Control of Asbestos Regulations 2012; Wildlife and Countryside Act 1981; Conservation of Habitats and Species Regulations 2017.

- Building Act 1984, Section 80 (notice of intended demolition to Havant Borough Council).

2. Project and Site Details

| Project & Site Details |  |
| --- | --- |
| Enquiry / Project No. | T2636 |
| Site address | Southleigh Park, Eastleigh Road, Havant, Hampshire PO9 2PE |
| Local planning authority | Havant Borough Council |
| Client | Bargate Homes Ltd — Will Harley, Commercial Manager |
| Principal Contractor | R Collard Ltd — for the demolition phase only, within a fenced site-within-a-site |
| Building to be demolished | Two-storey office building with glazed façade (1980s). Steel frame, block and beam floors, metal profile roof with felt covering. The single-storey link to the main house has already been removed, so the building is structurally independent. |
| Heritage context | Southleigh Park House, Grade II (List entry 1303673, listed 6 February 1984). The list entry expressly excludes the 1983 brick and glass corridor and the 1983 office building. The nearest listed structure is approx. 8 m from the gable end of the building being demolished. |
| Scope | Removal of non-licensed ACMs; soft strip; demolition of the superstructure; removal of ground floor slab (up to 300 mm) and foundations within the building footprint (up to 1.5 m); on-site crushing to 6F2; removal of all other materials. |
| Asbestos | Envirochem demolition survey J197641 (24/11/2020): one ACM — chrysotile gaskets to pipe flanges, ground floor east stairwell (non-licensed). First floor east stairs lift motor room not accessed and presumed to contain ACMs until inspected. |
| Recycled aggregate | Approx. 1,200 m³ of 6F2 produced on site and retained for Bargate’s reuse; surplus removed by RCL. |
| Programme | Approx. 4 weeks on site; demolition targeted for late 2026 (client to confirm start date). |
| Working hours | Monday–Friday 08:00–18:00; Saturday 08:00–13:00; no work on Sundays or Bank Holidays. |
| Site access | Eastleigh Road, via the existing Southleigh Park security gate. |
| Nearest A&E | Queen Alexandra Hospital, Southwick Hill Road, Cosham, Portsmouth PO6 3LY |

2.1 Site setting

Southleigh Park is a historic parkland estate of approx. 7.8 acres between Havant and Emsworth. Bargate Homes is converting the listed house and associated historic buildings into homes and building new houses across the estate; the main refurbishment works started in 2026 and are live. The office building to be demolished sits in mature parkland, with established trees around it and the listed house to the east (Figure 1).

Bargate Homes will fence off the demolition area to create a separate site within the main development, under RCL’s control as Principal Contractor for the demolition phase. Bargate will install tree protection before RCL takes possession.

3. Roles and Responsibilities

| Role | Name | Environmental responsibilities |
| --- | --- | --- |
| Managing Director | Gary Walsh | Overall accountability for RCL environmental performance and resourcing. |
| Contracts Manager | TBC at award | Owns this DEMP; approves RAMS; liaises with Bargate, the Conservation Officer and the Environmental Health Officer (EHO); reviews monitoring results weekly. |
| Site Manager (CCDO Gold) | Micky Watkins — TBC | Implements the DEMP daily; carries out inspections and records; first point of contact for complaints; authority to stop work. |
| HSE Advisor | Gerry Marchant | Audits compliance; investigates incidents; advises on monitoring and corrective action. |
| Asbestos removal | Collard in-house asbestos team | Non-licensed ACM removal under separate RAMS; hazardous waste consignment. |
| Plant operators / operatives | CPCS / NDTG certificated | Follow DEMP controls; run dust suppression; report spills, damage or unexpected finds immediately. |
| Client | Bargate Homes — Will Harley | Tree protection; ecological clearance; service disconnections (water retained for dust suppression); main-site coordination; heritage consultant / Conservation Officer liaison. |

4. Environmental Aspects and Sensitive Receptors

| Receptor | Distance | Potential effects and sensitivity |
| --- | --- | --- |
| Southleigh Park House and associated listed structures (Grade II) | Approx. 8 m from gable | Physical damage from debris, plant or falling material; vibration; dust soiling of flint and stonework. Highest sensitivity — controlling receptor. |
| Bargate main-site workforce and future occupiers of the converted house | 8 m + | Noise, dust and shared access. Managed through coordination with Bargate’s site team. |
| Mature parkland trees | Adjacent | Root compaction, canopy strike, pollution of root zones. Protected under Bargate’s arboricultural measures. |
| Bats and nesting birds | Building and trees | Disturbance or harm. Ecological clearance by Bargate before works start. |
| Residential properties off Eastleigh Road / Bartons Road | Off-site, beyond parkland | Noise and dust at distance; HGV movements and mud on the highway. |
| Site drainage and ground | On site | Spills of fuel or oil; silty run-off; contamination from redundant drainage. |
| Public highway | Haul route | Mud and debris on the road; traffic disruption. |

5. Protection of Listed Structures

The demolition is planned so that no machine, load or falling material is directed towards the listed structures at any stage. The measures below are the core of the planning response and will be written into the task RAMS.

5.1 Before demolition

- Condition survey. RCL will carry out a dated photographic condition survey of the elevations of the listed structures facing the works, including existing cracks and defects, and share it with Bargate before work starts. A repeat survey will be done at completion.

- Services and the former link. RCL will confirm that no services, drainage or structural connections remain between the office building and the listed house before any structural work.

5.2 During demolition

- Direction of working. Demolition starts at the existing entrance and works clockwise, finishing at the point closest to the listed structure. The building nearest the listed structure is therefore removed last, after the rest of the structure has created a working area.

- Hand removal of facing glazing. Glazing and cladding on the elevation facing the listed structure will be removed by hand from MEWPs, not by machine, so that no glass falls towards the listed building.

- Pull-in method. The steel frame nearest the listed structure will be cut with the shear and pulled inwards onto the footprint of the already demolished structure. The machine works from inside the footprint, facing away from the listed building, and does not slew loads over the protection zone.

- Controlled reduction. Members will be reduced piece by piece. Free-fall of large sections from height will not be permitted on the listed-building side.

- Low-vibration slab removal. Within 15 m of the listed structure, the ground slab and foundations will be broken by pulveriser, crusher bucket and lifting where practicable, with hydraulic breakers kept to a minimum and only used under vibration monitoring (Section 7).

- Dust suppression. Water spray will be directed at the point of demolition and away from the listed façade. Pressure-washing or saturation of historic flint and stone is not permitted.

- Supervision. A trained banksman will be present for all machine work within 15 m of the listed structure. The Site Manager will check the protection zone, screens and the façade at the start and end of each shift.

5.3 Damage or unexpected findings

If any damage to a listed structure is seen or suspected, work in that area stops straight away. The Site Manager makes the area safe, photographs and records the damage, and informs the Contracts Manager and Bargate the same day so that the Conservation Officer can be told. Work restarts only after the cause has been identified and the method revised. Any historic fabric found during demolition (for example remnants of earlier structures below ground) will be left in place and reported in the same way.

6. Noise Management

6.1 Assessment criteria

Noise is assessed using the BS 5228-1 Annex E ABC method. As no baseline survey is available, RCL has adopted the most onerous threshold — Category A, 65 dB LAeq,10h daytime at the façade of occupied dwellings. Saturday working (08:00–13:00) uses the same daytime threshold. RCL will serve the Section 80 demolition notice on Havant Borough Council. A Section 61 prior consent is not proposed given the short programme and distance to occupied dwellings, but RCL will comply fully with any Section 60 notice.

6.2 Predicted noise levels

Source levels are taken from BS 5228-1 Annex C: mobile crusher 90 dB LAeq at 10 m; tracked excavator demolishing structure 89 dB; hydraulic breaker 88 dB; pulveriser 87 dB; excavator loading and lorries 85 dB; MEWP 79 dB. The three loudest items working together give 94 dB at 10 m, reduced to about 92 dB at 10 m for a 60% on-time. This is a worst case: in practice crushing and heavy breaking will not run together for long periods.

| Distance from works | Unmitigated dB LAeq | With stockpile / screen (−5 dB) | With combined measures (−10 dB) |
| --- | --- | --- | --- |
| 25 m | 84 | 79 | 74 |
| 50 m | 78 | 73 | 68 |
| 100 m | 72 | 67 | 62 |
| 150 m | 68 | 63 | 58 |
| 200 m | 66 | 61 | 56 |
| 250 m | 64 | 59 | 54 |

Occupied dwellings off Eastleigh Road are separated from the works by parkland. With the measures below, the 65 dB threshold is met at 100 m or more. The converted listed house is not occupied during the demolition, and noise there is managed by coordinating with Bargate’s site team.

6.3 Control measures (Best Practicable Means)

- Work only within the stated hours. No crushing on Saturdays. Plant is not started or warmed up before 08:00.

- The mobile crusher is set up at the far side of the compound from the listed house and from occupied dwellings, screened by stockpiles of arisings.

- Modern, well-maintained plant with factory-fitted silencers and acoustic covers kept closed; plant switched off when not in use; broadband (“white noise”) reversing alarms.

- Pulverisers and shears used in preference to breakers wherever practicable.

- Avoid dropping materials from height; lorry loading kept away from the listed house.

- HGV arrivals managed so vehicles do not queue at the gate with engines running.

- The Site Manager will take hand-held sound level meter readings at the demolition boundary during the noisiest activities and record them, with extra checks if a complaint is received.

7. Vibration Management

Vibration is the main physical risk to the listed structure. RCL will use criteria for sensitive and historic buildings (DIN 4150-3, Line 3), which are lower than the BS 7385-2 values for modern structures. These limits are well below the levels at which cosmetic damage could occur.

| Location | Level (PPV) | Action |
| --- | --- | --- |
| Listed structure — façade nearest the works | Alert 2.0 mm/s | Alarm to Site Manager; review method; slow the rate of breaking or switch to pulveriser. |
| Listed structure — façade nearest the works | Action 3.0 mm/s | Stop vibration-generating activity in that area; inspect façade; Contracts Manager to agree revised method before restarting. |
| Occupied dwellings | Alert 1.0 mm/s | Human response threshold (BS 5228-2 Table B.1 — likely to cause complaint). Review method and inform residents. |

- Monitoring. A tri-axial vibration monitor with text/e-mail alarm will be fixed (by free-standing mount or non-damaging fixing agreed with Bargate) at the base of the listed structure façade nearest the works. It will run throughout structural demolition and slab/foundation removal within 30 m of the listed structure.

- Data are downloaded and reviewed weekly by the Contracts Manager, and immediately after any alert.

- Large members are lowered, not dropped. Drop heights are kept low and debris mats are used on the listed-building side.

- Breakers are not used within 15 m of the listed structure except where pulverising is not possible, and only with the monitor live.

8. Dust and Air Quality Management

8.1 Dust risk assessment (IAQM v2.2)

Step 1 — Screening. Human and heritage receptors lie within 250 m of the site boundary and within 50 m of the route used by site traffic, so a detailed assessment is required.

Step 2 — Dust emission magnitude. Demolition is classed as Large because concrete and masonry will be crushed and screened on site. The building is two storeys and under 12 m high, which on its own would be Medium. Earthworks are classed Small (working area under 18,000 m²). Trackout is classed Small (fewer than 20 HGV movements out per day; 2–3 a day expected).

Step 2 — Sensitivity of the area. Dust soiling: Medium — high-sensitivity receptors (the listed house, which is being converted to homes) but fewer than 10 within 20 m. Human health: Low — Havant Borough Council’s 2024 Air Quality Annual Status Report reports the borough as fully compliant with all air quality objectives, with background PM10 well below 24 µg/m³ and few receptors close to the works.

| Activity | Emission magnitude | Dust soiling risk | Human health risk | Overall |
| --- | --- | --- | --- | --- |
| Demolition | Large | Medium Risk | Medium Risk | Medium Risk |
| Earthworks | Small | Low Risk | Negligible | Low Risk |
| Construction | N/A | — | — | — |
| Trackout | Small | Negligible | Negligible | Negligible |

The site is therefore Medium Risk overall, driven by demolition and crushing. RCL will apply the IAQM Medium Risk measures to the whole site and add the heritage-specific controls in Section 5 at the listed-building interface. The works are programmed for the wetter winter months and a water supply is retained for dust suppression, so the residual effect is expected to be not significant.

8.2 Dust control measures

- Soft strip first. Complete the internal strip before structural demolition, so that only the frame, floors and roof remain.

- Water suppression. Water spray or mist from bowser or hydrant at the point of demolition, breaking, crushing and loading throughout the work. Water supply retained by Bargate for this purpose. Damping is directed away from the listed façade.

- Crusher. Operated with integral water suppression at the feed, crusher jaw and discharge conveyor, with conveyor drop heights kept to a minimum. The crusher is run under a Part B mobile plant permit (Environmental Permitting Regulations 2016), with Havant Borough Council notified before deployment, and the permit conditions are kept on site.

- Stockpiles. Kept away from the listed house and from trees, kept low, and damped or sheeted in dry or windy weather.

- Vehicles. 10 mph site speed limit on haul routes; loads sheeted; no idling.

- No burning of any material on site.

- Weather. In dry, windy conditions (sustained wind towards the listed house or dwellings), the Site Manager will increase damping or stop dusty activities until conditions improve.

9. Waste and Materials Management

Waste is managed under the waste hierarchy and the Duty of Care. Materials are separated at source during soft strip and demolition to maximise reuse and recycling. RCL targets at least 95% diversion from landfill (by weight) for non-hazardous demolition waste.

| Waste stream | EWC code (indicative) | Destination |
| --- | --- | --- |
| Concrete, block and beam, masonry | 17 01 01 / 17 01 07 | Crushed on site to 6F2 (SHW Series 600) — approx. 1,200 m³ retained for reuse; surplus removed by RCL. |
| Structural steel and metal roof sheeting | 17 04 05 | Segregated by magnet and shear; to metal recycler. |
| Glazing | 17 02 02 | Segregated for glass recycling. |
| Timber | 17 02 01 | Segregated for recycling / biomass recovery. |
| Plasterboard / gypsum | 17 08 02 | Segregated and kept dry for gypsum recycling. |
| Roofing felt | 17 03 02 | Off-site disposal or recovery (sampled for coal tar before removal). |
| Mixed soft strip waste | 17 09 04 | To a permitted waste transfer station for sorting and recovery. |
| Asbestos (chrysotile gaskets) | 17 06 05* | Hazardous waste — double-bagged, consigned to a permitted hazardous waste facility. |
| WEEE and fluorescent tubes | 20 01 35* / 20 01 21* | Hazardous waste — to permitted recovery facility. |

- Production of 6F2 follows the WRAP Quality Protocol for aggregates from inert waste, with the appropriate waste exemption or EA authorisation for crushing in place before processing. Grading tests are kept on file.

- Arisings to be crushed are checked for contaminants (gypsum, timber, plastics, asbestos) and rejected material is removed.

- Every load is covered by a waste transfer note or hazardous waste consignment note. Carriers and receiving sites are checked for valid registration and permits. All records are kept on site and handed over at completion.

- Existing loose items and furniture are assumed cleared by others before RCL starts.

10. Pollution Prevention, Drainage and Ground

- Fuel is held in a double-skinned, bunded bowser (110% capacity), locked when not in use, in a set location more than 10 m from drains and outside tree root protection areas.

- Refuelling is done by trained operatives with drip trays and a spill kit at hand. Plant is checked daily for leaks.

- Spill kits are kept at the fuel point, on each excavator and at the crusher. Operatives are briefed on their use.

- Surface water gullies near the works are protected with drain covers or filter socks; silty water is not allowed to enter drains.

- Redundant drainage is located, capped and removed with the foundations. No wash-out of concrete or plant to ground or drains.

- Underground tanks, contaminated ground or unexpected materials (e.g. odours, staining, buried waste or suspected asbestos) are not part of the tender allowance. If found, work stops in that area and the find is reported to Bargate (Section 15).

- Voids left by foundations are backfilled with 6F2 or left as required by Bargate, and the site is left clean.

11. Trees and Ecology

11.1 Trees

- Bargate will install tree protection fencing to its arboricultural method statement before RCL takes possession. RCL will not move or alter this fencing.

- No plant tracking, storage, stockpiling, fuel, crusher set-up or wash-out inside root protection areas.

- Machine reach and slewing near tree canopies is supervised by a banksman; no material is dropped through or onto canopies.

- No fires and no attaching of signs or equipment to trees.

- Any damage to trees or protective fencing is reported to Bargate on the same day.

11.2 Ecology

- Demolition will not start until Bargate confirms that any required ecological surveys, bat checks and mitigation (including any licence) are complete. Ecology and bat mitigation are outside RCL’s scope.

- Works are programmed for late 2026, outside the main bird nesting season (March–August). If a nest or a bat is found at any time, work in that area stops and Bargate’s ecologist is contacted.

- Site lighting, if needed in winter, is kept to the working area and directed downward, away from tree lines.

- Excavations are covered or fitted with a ramp overnight so animals cannot become trapped.

12. Traffic, Highways and Logistics

- Access. From Eastleigh Road via the existing Southleigh Park security gate. RCL’s compound is a separate site within Bargate’s site, with its own Heras perimeter and signage.

- HGV route. Eastleigh Road – Bartons Road – Petersfield Road – through Havant to the A27. Drivers are briefed on the route and given a route map.

- Volumes. About 2–3 HGV movements a day over the 4-week programme, as most concrete and masonry stays on site as 6F2. Plant delivery and collection is by low-loader at mobilisation and demobilisation.

- Timing. Deliveries and collections are scheduled to avoid peak traffic and Bargate’s main deliveries where practicable, and coordinated with Bargate’s site team.

- Highway cleanliness. Wheel cleaning on site; road sweeper on call to keep the public highway clean; all loads sheeted or netted.

- On site. All vehicle movements are controlled by a trained banksman, pedestrian and vehicle routes are segregated, and a 10 mph speed limit applies. Vehicles do not park on the public highway.

13. Community and Stakeholder Liaison

- A site notice board at the gate shows the RCL Site Manager’s name and 24-hour contact number, working hours and the Collard office number (01252 844688).

- Bargate and its site team are given at least 7 days’ notice of the start date, the crushing period and any planned noisy activities. RCL attends Bargate coordination meetings as required.

- Where Bargate asks, RCL will provide a letter to neighbours explaining the works, hours and contact details before work starts.

- Complaints. Every complaint is logged with date, time, nature and action taken. The Site Manager investigates on the same day, takes corrective action and responds to the complainant (or via Bargate) within 24 hours. Complaints are reviewed by the Contracts Manager weekly.

- Any contact from Havant Borough Council (EHO, Conservation Officer or Building Control) is passed to the Contracts Manager and Bargate the same day.

14. Monitoring, Inspection and Records

| What | Frequency | By | Record |
| --- | --- | --- | --- |
| Protection zone, screens and listed façade | Start and end of each shift | Site Manager | Daily environmental checklist |
| Vibration at listed structure | Continuous during demolition / slab removal within 30 m | Monitor; reviewed by Contracts Manager | Downloaded data; alert log |
| Dust — visual on-site and boundary inspection | At least twice daily; more in dry or windy weather | Site Manager | Daily checklist; dust log |
| Noise — hand-held spot readings | During the noisiest activities and after any complaint | Site Manager | Noise log |
| Water suppression working | Daily | Plant operators | Pre-use plant check |
| Highway cleanliness and wheel wash | Daily | Site Manager | Daily checklist |
| Tree protection fencing | Daily | Site Manager | Daily checklist |
| Fuel storage, spill kits, drain protection | Daily | Site Manager | Daily checklist |
| Waste documentation | Each load | Site Manager | Waste transfer / consignment notes |
| Environmental audit | At least once during the works | HSE Advisor | Audit report |

Records are kept on site and made available to Bargate and Havant Borough Council on request. A close-out pack is provided at completion, including waste records, the 6F2 test results, the post-works condition survey of the listed structures and the vibration monitoring summary.

15. Environmental Incidents and Emergency Response

| Incident | Immediate response |
| --- | --- |
| Damage to listed structure | Stop work in the area; make safe; photograph; inform Contracts Manager and Bargate the same day; do not attempt repairs to historic fabric. |
| Vibration action level exceeded | Stop the activity; inspect the façade; record; revise method before restarting. |
| Visible dust leaving site | Increase suppression; stop the activity if dust continues; record cause and action. |
| Fuel or oil spill | Stop the source; contain with spill kit; protect drains; bag contaminated material as hazardous waste; report to Site Manager. Serious spills reported to the Environment Agency (0800 80 70 60). |
| Suspected asbestos found | Stop work; isolate the area; Collard asbestos team to assess and sample; do not restart until cleared. |
| Contamination, tanks or unknown buried items | Stop work in that area; fence off; inform Bargate; agree investigation and handling. |
| Bats or nesting birds found | Stop work in that area; leave undisturbed; contact Bargate’s ecologist. |
| Damage to trees or tree protection | Stop; report to Bargate and its arboriculturist the same day. |

All environmental incidents and near misses are recorded and investigated by the HSE Advisor, and lessons learned are briefed to the site team. Emergency arrangements (first aid, assembly point and route to Queen Alexandra Hospital) are as set out in the Demolition Method Statement.

16. Training and Communication

- Everyone on site receives a site induction covering this DEMP, including the heritage protection zone, working hours, dust and noise controls, tree protection, spill response and complaint handling.

- Toolbox talks are given before the start of each main phase: soft strip, demolition near the listed structure, crushing, and slab and foundation removal.

- Plant operators hold valid CPCS / NDTG cards for the plant they operate. The crusher is run by an operator trained in its dust controls.

- The site is managed by CCDO Gold qualified personnel as a minimum.

- This DEMP is reviewed if the method, programme or site conditions change, or after any incident or substantiated complaint, and reissued with a new revision number.

Appendix A – Pre-commencement Confirmations

The following must be confirmed before RCL starts work on site. They do not change the controls in this DEMP.

| No. | Item | Responsibility | Status |
| --- | --- | --- | --- |
| 1 | Start date and programme confirmed. | Bargate / RCL | Open |
| 2 | Tree protection installed to Bargate’s arboricultural method statement. | Bargate | Open |
| 3 | Ecological clearance confirmed, including bat checks and any licence. | Bargate | Open |
| 4 | All services disconnected (written confirmation); water supply retained for dust suppression. | Bargate | Open |
| 5 | Lift motor room (first floor east stairs) inspected for asbestos; non-licensed ACMs removed. | RCL | Open |
| 6 | Photographic condition survey of the listed structures facing the works. | RCL | Open |
| 7 | Protection to listed façade openings agreed with Bargate / heritage advisor. | Bargate / RCL | Open |
| 8 | Vibration monitor installed and alarms tested. | RCL | Open |
| 9 | Section 80 notice served on Havant Borough Council; mobile crusher deployment notified; waste exemption / authorisation for crushing in place. | RCL | Open |
| 10 | Nearest occupied dwellings identified and confirmed for the noise and dust receptor list. | RCL / Bargate | Open |